Unit – I

Basic structure of computers and Computer arithmetic

Lecture1

Lecture-2

Lecture-3

Lecture-4

Lecture-5

Characteristics of floating point numbers:

The characteristics are 1. Precision  2. Gap 3. Range

Precision:

It characterizes how precise a floating point value can be. It is defined as the number of bits in the significand. The greater the number of bits in the significand, the greater is the CPU’s precision and the more precise is it’s value. Many CPUs have 2 representations for floating point numbers. They are called single precision and double precision here double precision has twice the number of bits.

Gap:

The gap is the difference between two adjacent values. It’s value depends on the value of the exponent.

Take the number: X = .10111010 * 23 .

It’s adjacent values are :  .10111001 * 23   and  .10111011* 23 .

Each number produce a gap of .00000001* 23 .

In general the gap for floating point value X can be expressed as  2( Xe-precision)

Range:

The range of a  floating point representation is bounded by it’s smallest and largest possible values.

Overflow and underflow;

Overflow occurs when an operation produces a result that can not be stored in computers’s floating point registers. Underflow occurs when an operation produces a result between zero and either the positive or negative smallest possible value.

IEEE 754 Floating point standard:

This standard specifies 2 precision for floating point numbers which are called single precision and double precision floating point representations.

Single Precision Format:

This format has 32 bits. 1 bit for sign; 8 bits for the exponent; 23 for the significand. The significand also includes an implied 1 to the left of its radix point( except for special values and denormalized numbers).

Error Detection codes:

Information is stored as binary codes and are transmitted by serial or parallel communication. During transmission noise is added to the signal and it may change binary bits in the code from 1 to 0, and vice versa. An error detection code is a binary code that detects digital errors during transmission. The detected errors can not be corrected but their presence is indicated.

Parity bit:

The most common error detection code used is the parity bit. A parity bit is an extra bit included with a binary message to make the total number of 1’s either odd or even. If the message consists of n bits , then the error detection code consists of n+1 bits. If the bit added to the message makes the sum of 1’s odd in the error detection code, then the scheme is called odd-parity. If the sum of bits is even , the scheme is called even parity scheme.

| Message xyz | P(odd) | P(even) | Error detection code, odd parity | Error detection code, even parity |
| --- | --- | --- | --- | --- |
| 000 | 1 | 0 | 0001 | 0000 |
| 001 | 0 | 1 | 0010 | 0011 |
| 010 | 0 | 1 | 0100 | 0101 |
| 011 | 1 | 0 | 0111 | 0110 |
| 100 | 0 | 1 | 1000 | 1001 |
| 101 | 1 | 0 | 1011 | 1011 |
| 110 | 1 | 0 | 1101 | 1101 |
| 111 | 0 | 1 | 1110 | 1111 |

Parity Generator and Parity Checker:

X

Y

z

odd Parity out

Parity Checker:

X

Y

z

Parity out from generator

Error

Indication

The circuit arrangement checks the occurrence of error any odd number of times. An even number of errors is not detected.

We note that P(even) function is the exclusive –OR x,y,z because it is equal to 1 when either one or all 3 of the variables are equal to 1. The P(odd) function is the complement of the P(even) function.

Assume at the sending end  the message bits and odd parity bit is generated. The EX-OR gates generate P(even ) function and to generate P(odd), the complement of P(even) is used.

The 4 bits transmitted has an odd number of I’s. If an error occurs during transmission, then the number of 1’s become even. Hence parity checker checks for even parity.

Lecture-6

COMPUTER ARITHMETIC:

Addition, subtraction, multiplication are the four basic arithmetic operations. Using these operations other arithmetic functions can be formulated and scientific problems can be solved by numerical analysis methods.

Arithmetic Processor:

It is the part of a processor unit that executes arithmetic operations. The arithmetic instructions definitions specify the data type that should be present in the registers used . The arithmetic instruction may specify binary or decimal data and in each case the data may be in fixed-point or floating point form.

Fixed point numbers may represent integers or fractions. The negative numbers may be in signed-magnitude or signed- complement representation. The arithmetic processor is very simple if only a binary fixed point add instruction is included. It would be more complicated if it includes all four arithmetic operations for binary and decimal data in fixed and floating point representations.

Algorithm:

Algorithm can be defined as a finite number of well defined procedural steps  to solve a problem. Usually, an algorithm  will contain a number of procedural steps which are dependent on results of previous steps. A convenient method for presenting an algorithm is a flowchart which consists of rectangular and diamond –shaped  boxes. The computational steps are specified in the rectangular boxes and the decision steps are indicated inside diamond-shaped boxes from which 2 or more alternate path emerge.

Addition and Subtraction:

3 ways of representing negative fixed point binary numbers:

- Signed-magnitude representation---- used for the representation of mantissa  for floating point operations by most computers.

- Signed-1’s complement

- Signed -2’s complement—Most computers use this form for performing arithmetic operation with integers

Addition and subtraction algorithm  for signed-magnitude data

Let the magnitude of two numbers be A & B. When signed numbers are added or subtracted, there are 4 different conditions to be considered for each addition and subtraction depending on the sign of the numbers. The conditions are listed in the table below. The table shows the operation to be performed with magnitude(addition or subtraction) are indicated for different conditions.

| Sl.No | Operation | Add Magnitudes | Subtract magnitudes |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | When A> B | When A< B | When A=B |
| 1 | ( +A ) + (+B ) | + ( A + B ) |  |  |  |
| 2 | ( +A ) + (-B ) |  | +( A-B ) | -( B-A ) | +( A-B ) |
| 3 | ( -A ) + (+B ) |  | -( A-B ) | +( B-A ) | +( A-B ) |
| 4 | ( -A ) + (-B ) | - ( A + B ) |  |  |  |
| 5 | ( +A ) - (+B ) |  | +( A-B ) | -( B-A ) | +( A-B ) |
| 6 | ( +A ) - (-B ) | + ( A + B ) |  |  |  |
| 7 | ( -A ) - (+B ) | - ( A + B ) |  |  |  |
| 8 | ( -A ) - (-B ) |  | -( A-B ) | +( B-A ) | +( A-B ) |

The last column is needed to prevent a negative zero. In other words, when two equal numbers are subtracted, the result should be +0 not -0.

The algorithm for addition and subtraction ( from the table above):

Addition Algorithm:

When the signs of A and B are identical, add two magnitudes and attach the sign of A to the result. When the sign of A and B are different, compare the magnitudes  and subtract the smaller number from the larger. Choose the sign of the result to be the same as A if A>B or the complement of sign of A if A < B. If the two magnitudes are equal,  subtract B from A and make te sign of the result positive.

Subtraction algorithm:

When the signs of A and B are different, add two magnitudes and attach the sign of A to the result. When the sign of A and B are identical, compare the magnitudes  and subtract the smaller number from the larger. Choose the sign of the result to be the same as A if A>B or the complement of sign of A if A < B. If the two magnitudes are equal,  subtract B from A and make te sign of the result positive.

Hardware Implementation:

Let A and B are two registers that hold the numbers.

AS and BS  are 2, flip-flops that hold sign of corresponding numbers. The result is stored In A and AS .and thus they form Accumulator register.

We need to perform  micro operation, A+ B and hence  a parallel adder.

A comparator is needed to establish if A> B, A=B, or A<B.

We need to perform micro operations  A-B and B-A and hence two parallel subtractor.

An exclusive OR gate can be used to determine the sign relationship, that is, equal or not.

Thus the hardware components required are a magnitude comparator, an adder, and two subtractors.

Reduction of hardware by using different procedure:

- We know subtraction can be done by complement and add.

- The result of comparison can be determined  from the end carry after the subtraction.

We find An adder and a complementer  can do subtraction and comparison if 2’s complement is used for subtraction.

Hardware forsigned-magnitude addition and subtraction:

AVF    Add overflow flip flop.  It hold the overflow bit when A & B are added.

Flip flop E—Output carry is transferred to E. It can be checked to see the relative magnitudes of the two numbers.

A-B = A +( -B )= Adding a and 2’s complement of B.

The A register provides other micro operations that may be needed when the sequence of steps in the algorithm is specified.

The complementer Passes the contents of B or the complement of B to the Parallel Adder depending on the state of the mode control B. It consists of EX-OR gates and the parallel adder consists of full adder circuits. The M signal is also applied to the input carry of the adder.

When input carry  M=0, the sum of full adder is A +B.  When M=1, S = A + B’ +1=  A – B

Hardware algorithm:

Flow Chart for Add and Subtract operations:

The EX-OR gate provides 0 as output when the signs are identical. It is 1 when the signs are different.

A + B is computed for the following and the sum is stored in EA:

- When the signs are same and addition operation is required.

- When the signs are different and subtract operation is required.

The carry in E after addition indicates an overflow if it is 1 and it is transferred to AVF, the addoverflow flag

A-B = A+ B’+1 computed for the following:

- When the signs are different and addition operation is required.

- When the signs are same and subtract operation is required.

No overflow can  occur if the numbers are subtracted and hence AVF is cleared to Zero.

[ the subtraction of 2 n-digit un signed numbers M-N ( N≠0) in base r can be done as follows:

- Add minuend M to thee r’s complement of the subtrahend N. This performs M-N  +rn .

- If M ≥ N, The sum will produce an end carry rnwhich is discarded, and what is left is the result M-N.

- If M< N, the sum does not produce an end carry and is equal to  rn–( N-M ), which is the r’s complement of the sum and place a negative sign in front.]

A 1 in E indicates that A ≥ B and the number in A is the correct result.

If this number in A is zero, the sign AS must be made positive to avoid a negative zero.

A 0 in E indicates that A< B. For this case it is necessary to take the 2’s complement of the value in A.

In the algorithm shown in flow chart, it is assumed that A register has circuits for micro operations complement and increment. Hence two complement of value in A is obtained in 2, micro operations. In other paths of the flow chart , the sign of the result is the same as the sign of A, so no change in AS is required.

However When A  < B, the sign of the result is the complement of original sign of A.

Hence The complement of AS stored in  AS.

Final Result: AS A

Flow chart for ADD and Subtract operations:

Addition and Subtraction with signed-2’s complement Data.:

Arithmetic Addition:

This method does not need a comparison or subtraction but only addition and complementation. The procedure is as below:

- Represent the negative numbers in 2’s complement form.

- Add the two numbers including the sign bits and discard any carry out of sign bit position.

- The overflow bit V is set to 1 if there is a carry into sign bit and no carry out of sign bit or if there is a no carry into sign bit and a carry out of sign bit. Otherwise it is set to zero.

- If the result is negative, take the 2’s complement of the result to get a correct negative result.

Arithmetic Subtraction:

- Represent the negative numbers in 2’s complement form.

- Take the 2’s complement of the subtrahend including the sign bit and add it to the minuend including the sign bit.

- The overflow bit V is set to 1 if there is a carry into sign bit and no carry out of sign bit or if there is a no carry into sign bit and a carry out of sign bit. Otherwise it is set to zero.

- Discard the carry out of the sign bit position.

Note: A subtraction operation can be changed to an addition operation if the sign of the subtrahend is changed.

BR Register

V

AC Register

Complementer&Parallel Adder

Overflow

Fig: Hardware for Signed 2/s complement for addition/ subtractioin.

Lecture-7

Multiplication Algorithm:

Hardware implementation of multiplication of numbers in signed – magnitude form:

- A adder is provided to add two binary numbers and the partial product is accumulated in a register.

- Instead of shifting the multiplicand to the left, the partial product is shifted to the right, which result in leaving the partial product and the multiplicand in the required relative positions.

- When the corresponding bit of the multiplier is zero, there is no need to add all zeros to the partial product, since it will not alter it’s value.

The hardware consists of 4 flipflops, 3 registers, one sequence counter , an adder and complementer.

Q register&QS  flip flop           :     contains multiplier & Its sign

Sequence counter                    :       It is set to a value equal to the number of bits in the multiplier

B Register& BS flipflop            :       It contains the multiplicand,& its sign

A Register, E Flip flop               :      Initialized to ‘ 0’.   AS denotes sign of partial product

EA Register                                :    hold partial product, with carry generated in addition being shifted to E .

Qn                    :    Rightmost bit of the multiplier;  AQ : will contain the final product.

As AQ represent product register, both AS  QSrepresent the sign of the partial product or product.

The number to be multiplied are stores in memory as n bit sign magnitude numbers and when transferred to register msb bit go to sign flipflop and remaining n-1 bits go to registers. Hence SC is initially set to n-1.

Let the lower order bit of the multiplier in Qntested.

If it is 1, the multiplicand in B is added to the present partial product in A.

If it is a ‘0’, nothing is done. Register EAQ is then shifted once to the right to form the new partial product. The sequence counter is decremented by 1 and it’s new  value checked. If it is not equal to zero, the process is repeated and a new partial product is formed. The process stops when SC = 0.

The final product is available in both A and Q, with A holding the most significant bits and Q holding the least significant bits.

Flowchart for multiply operation:

Numerical Example for the above algorithm:

| Multiplicand B= 10111 | E | A | Q | SC |
| --- | --- | --- | --- | --- |
| Multiplier in Q Qn =1;add B First Partial Product Shift Right EAQ | 0 0 0 | 00000 10111 10111 01011 | 10011 11001 | 101 100 |
| Qn =1;add B Second Partial Product Shift Right EAQ | 1 0 | 10111 00010 10001 | 01100 | 011 |
| Qn =0; Shift Right EAQ | 0 | 01000 | 10110 | 010 |
| Qn =0; Shift Right EAQ | 0 | 00100 | 01011 | 001 |
| Qn =1;add B Fifth Partial Product Shift Right EAQ | 0 0 | 10111 11011 01101 | 10101 | 000 |
| Final Product in AQ AQ = 0110110101 |  |  |  |  |

Lecture-8

Booth Multiplication Algorithm:

Multiplication of signed- 2’s complement  integers:

This algorithm uses the following facts.

- A string of 0’s in the multiplier requires no addition but just shifting.

- A string of 1’s in the multiplier from bit weight 2k  to weight 2m  can be treated as 2k+1  - 2m.

Example: Consider the binary number: 001110( +14 )

The number has a string of 1’s from 23  to 21 .  Hence k = 3 and m= 1. As other bits are 0’s, the number can be represented as 2k+1  - 2m  = 24 – 21  = 16-2 =  14. Therefore the multiplication M * 14 , where M is the multiplicand and 14 the multiplier can be done as Mx 24 –M x 21.

This can be achieved  by shifting binary multiplicand M four times to the left and subtracting M shifted left once which is equal to (Mx 24 –M x 21. ).

Shifting and addition/subtraction rules for multiplicand in Booth’s Algorithm:

- The multiplicand is subtracted from the partial product upon encountering the first least significand 1 in a string of I’s in the multiplier.

- The multiplicand is added to the partial product upon encountering the first 0 ( provided that there was a previous 1)in a string of 0’s in the multiplier.

- The partial product does not change when the multiplier bit is identical to the previous multiplier bit

Hardware Implementation of Booth Algorithm:

Note: Sign bit is not separated from register. QR register contains the multiplier register and Qnrepresent the least significant bit of the multiplier in QR. Qn+1 is an extra flip flop appended to QR to facilitate a double bit inspection of the multiplier.

AC register and appended  Qn+1  are initially cleared to 0.

Sequence counter Sc is set to the number n which is equal to the number of bits of bits In the multiplier.

QnQn+1 are to successive bits in the multiplier

Example for multiplication using Boot h algorithm:

| QnQn+1 | BR = 1011 ,BR'+1 = 01001 | AC | QR | Qn+1 | SC |
| --- | --- | --- | --- | --- | --- |
| 10 | Initial Subtract BR ashr | 00000 01001 01001 00100 | 10011 11001 | 0 1 | 101 100 |
| 11 | ashr | 00010 | 01100 | 1 | 011 |
| 01 | Add BR ashr | 10111 11001 11100 | 10110 | 0 | 010 |
| 00 | ashr | 11110 | 01011 | 0 | 001 |
| 10 | Subtract BR Ashr | 01001 00111 00011 | 10101 | 1 | 000 |

Algorithm in flowchart for multiplication of signed  2’s complement numbers.

Array Multiplier:

2 -bit by 2- bit Array Multiplier:

Multiplicand bits are b1 and b0 .Multiplier bits are a1 and a0 .The first partial product is obtained by multiplying a0 by b1 b0 . The bit multiplication is implemented by AND gate. First partial product is made by two AND gates. Second partial product is made by two AND gates. The two partial products are added with two half adder circuits.

Combinational circuit binary multiplier:

A bit of the multiplier is ANDed with each bit of the multiplicand in as many levels as there bits in the multiplier. The binary output in each level of the AND gates is added in parallel with the partial product of the previous level to form a ne partial product. The last level produces the product. For j multiplier and k multiplicand bits, we need j*k AND Gates and (j-1)*k bit adders to ptoduce a product of j+k bits.

4- bit by 3-bit Array Multiplier:

Lecture-9

Division Algorithms:

Division Process for division of fixed point binary number in signed –magnitude representation:

Let dividend A consists of 10 bits and divisor B consists of 5 bits.

- Compare the 5 most significant bits of the dividend with that of divisor.

- If the 5 bit number is smaller than divisor B, then take 6 bits of the dividend and compare with the 5 bit divisor.

- The 6 bit number is greater  than divisor B. Hence place a 1 for the quotient bit in the sixth position above the dividend. Shift the divisor once to the right and subtracted from the dividend. The difference is called partial remainder.

- Repeat the process with the partial remainder and divisor. If the partial remainder is equal or greater than or equal to the divisor, the quotient bit is equal to 1.The divisor is then shifted right and subtracted from the partial remainder. If the partial remainder is small than the divisor, then the  quotient bit is zero and no subtraction is needed. The divisor is shifted once to the right in any case,.

Hardware Implementation of division for signed magnitude fixed point numbers:

To implement division using a digital computer, the process is changed slightly for convenience.

- Instead of shifting the divisor to the right, the dividend or the partial remainder, is shifted to the left so as to leave the two numbers in the required relative position.

- Subtraction may be achieved by adding A (dividend)to the 2’s complement of B(divisor). The information about the relative magnitude is then available from end carry.

- Register EAQ is now shifted to the left with 0 inserted into Qn and the previous value of E is lost..

- The divisor is stored in B register and the double length dividend  is stored in registers A and Q.

- The dividend is shifted to the left and the divisor is subtracted by adding  it’s 2’s complement value.

- If E= 1, it signifies that A ≥ B.A quotient bit is inserted into Qnand the partial remainder is shifted to the left to repeat the process.

- If E = 0, it signifies that A  < B so the quotient Qn remains 0( inserted during the shift). The value of B is then added to restore the partial remainder in A to its previous value. The partial remainder is shifted to the left and the process is repeated again until all 5 quotient bits are formed.

- At the end Q contains the quotient and A the remainder. If the sign of dividend and divisor are alike, the quotient is positive and if unalike, it is negative. The sign of the remainder is the same as dividend.

Sequence Counter( SC)

B Register

QS

Complementer and parallel adder

E

Q Register

A Register

AS

Qn

0

Hardware for implementing division of fixed point signed- Magnitude Numbers

Example of Binary division with digital hardware:   Divisor B = 10001,     B     + 1  = 01111

|  |  | E | A | Q | SC |
| --- | --- | --- | --- | --- | --- |
|  | Dividend: |  | 01110 | 00000 | 5 |
|  | Shl EAQ |  | 11100 | 00000 |  |
|  | Add , B + 1 |  | 01111 |  |  |
|  | E = 1 | 1 | 01011 |  |  |
|  | Set Qn= 1 | 1 | 01011 | 00001 | 4 |
|  | Shl EAQ | 0 | 10110 | 00010 |  |
|  | Add , B + 1 |  | 01111 |  |  |
|  | E = 1 | 1 | 00101 |  |  |
|  | Set Qn= 1 | 1 | 00101 | 00011 | 3 |
|  | Shl EAQ | 0 | 01010 | 00110 |  |
|  | Add , B + 1 |  | 01111 |  |  |
|  | E= 0; Leave Qn= 0 | 0 | 11001 | 00110 |  |
|  | Add B |  | 10001 |  |  |
|  | Restore remainder | 1 | 01010 |  | 2 |
|  | Shl EAQ | 0 | 10100 | 01100 |  |
|  | Add , B + 1 |  | 01111 |  |  |
|  | E = 1 | 1 | 00011 |  |  |
|  | Set Qn= 1 | 1 | 00011 | 01101 | 1 |
|  | Shl EAQ | 0 | 00110 | 11010 |  |
|  | Add , B + 1 |  | 01111 |  |  |
|  | E= 0; Leave Qn= 0 | 0 | 10101 | 11010 |  |
|  | Add B |  | 10001 |  |  |
|  | Restore remainder | 1 | 00110 | 11010 | 0 |
|  | Neglect E |  |  |  |  |
|  | Remainder in A |  | 00110 | 11010 |  |
|  | Quotient in Q |  |  |  |  |

Divide overflow:

When the dividend is twice as long as the divisor, the condition for overflow can be stated as follows:

A divide-overflow condition occurs if the higher order half bits of the dividend constitute a number greater than or equal to the divisor. If the divisor is zero, then the dividend will definitely be greater than or equal to divisor. Hence divide overflow condition occurs and hence the divide-overflow –flip flop will be set. Let the flip flop be called DVF.

Handling DVF:

- Check if DVF is set after each divide instruction. If DVF is set, then the program branches to a subroutine that takes corrective measures such as rescaling the data to avoid overflow.

- An interrupt is generated if DVF is set. The interrupt causes the processor to suspend the current program and branch to interrupt service routine to take corrective measure. The most common corrective measure is to remove the program  and type an error message that explains the reasons.

- The divide overflow can be handled  very simply if the numbers are represented in floating point representation.

Flow chart for divide operation:

Assumption:

Operands are transferred from memory to registers as n bit words.n-1 bit form magnitude and 1 bit shows the sign.

A divide overflow condition is tested by subtracting the divisor in B from half of the bits of dividend stored in A. If vA ≥ B, the DVF is set and the operation is terminated prematurely. If A < B, no DVF occurs and so the value of dividend is restored by adding B to A.

The division of the magnitudes starts by shifting the dividend in AQ to the left, with the higher order bit shifted into E. If the bit shifted into E is 1, we know that EA is greater than B because EA consists of a 1 followed by n-1 bits while B consists of only n-1 bits. In this case, B must be subtracted from EA and 1 inserted into Qn for the quotient bit.  Since register A is missing the higher order bit of the dividend (which is in E), it’s value is EA – 2n-1  . Adding to this value the 2’s complement of B results in

(EA-2n-1 ) + ( 2n-1 –B )= E-B. The carry from the addition is not transferred to E if we want E to remain a 1.

If the shift left operation inserts a zero into E, the divisor is subtracted by adding it’s 2’s complement value and the carry is transferred into E. If E = 1, it signifies that A ≥ B and hence Qn is set to 1. If E = 0, it signifies that A < B and the original number is restored by adding B to A. In the latter case we leave a 0 in Qn .( 0 was inserted during the shift).

This process is repeated again with register A holding the partial remainder. After n-1 times, the quotient magnitude is formed in the register Q and the remainder is found in register A.

      - Lecture-10

Floating-Point Addition and Subtraction

Floating-Point Multiplication

Floating-Point Division

Lecture-11

Lecture-12

Lecture-13

Test Questions

Fill in the blanks type of questions <Minimum of ten>

- The decimal representation for hex number F3 is                                         .

- The binary equivalent for the decimal number  41.6875 is

- The BCD code for the decimal number  248 is                                                                   .

- For a given number N in base r having n digits, the (r-1)’s complement of N is defined as

- The 10’s complement of a decimal number is obtained by adding           to the 9’s complement value.

- When 2 unsigned numbers are added, an overflow is detected from the                                         of the most significant position.

- An overflow for addition/ subtraction of two signed numbers  is detected when  the carry into the sign bit position and carry out of the sign bit position are                                          .

- Booth multiplication algorithm is followed when the binary integers are represented in

- When Booth algorithm is used for multiplication, the partial product does not change when the multiplier .is identical to the previous multiplier            .

- Floating point multiplication and division do not require an alignment of the                                   .

Answers: ( 1).           243                (2)   101001.1011                 (3)  0010  0100   1000

(4)        (  rn-1)-N           (5)    1              (6)    carry out         (7)  not equal

(8) signed 2’s complement representation for negative integers.    (9)   bit, bit

(10) mantissa

Multiple choice questions<Minimum of ten>

1. Floating point representation is used to store

(A) Boolean values (B) whole numbers (C) real integers (D) integers

Ans: C

2. In computers, subtraction is generally carried out by

(A) 9’s complement (B) 10’s complement (C) 1’s complement (D) 2’s complement

Ans: D

3. The circuit used to store one bit of data is known as

(A) Register (B) Encoder (C) Decoder (D) Flip Flop

Ans: D

4. Which of the following is not a weighted code?

(A) Decimal Number system (B) Excess 3-cod

(C) Binary number System (D) None of these

Ans: B

5. Assembly language

(A) uses alphabetic codes in place of binary numbers used in machine language

(B) is the easiest language to write programs

(C) need not be translated into machine language

(D) None of these

Ans: A

6. The multiplicand register & multiplier register of a hardware circuit implementing booth's algorithm have (11101) & (1100). The result shall be

(A) (812) 10 (B) (-12) 10 (C) (12) 10 (D) (-812) 10

Ans: A

7. What characteristic of RAM memory makes it not suitable for permanent storage?

(A) too slow (B) unreliable (C) it is volatile (D) too bulky

Ans: C

8. (2FAOC) 16 is equivalent to

(A) (195 084) 10 (B) (001011111010 0000 1100) 2 (C) Both (A) and (B) (D) None of these

Ans: B

9. The average time required to reach a storage location in memory and obtain its contents is called the

(A) seek time (B) turnaround time (C) access time (D) transfer time

Ans: C

10. In signed-magnitude binary division, if the dividend is (11100) 2 and divisor is (10011) 2 then the result is

(A) (00100) 2 (B) (10100) 2 (C) (11001) 2 (D) (01100) 2

- Ans: B

Fill the blank with true or false.

- EEPROM comes under volatile memory category.

- Thumb drive or pen drive is semiconductor memory.

- The control unit generates the appropriate signal at the right moment.

- While executing a program, CPU brings instruction and data from disk memory.

- A memory module of capacity 16 * 4 , indicates a storage of 128 bits.

- A memory module of capacity of 1024 locations, the required address bus size is 10.

- The program counter PC is used to store the address of the next instruction to be fetched from Accumulator.                                                                                                                                                       .

- For n-bit signed integer, the range of numbers that can be represented is – 2n-1  to 2n+1 .

- Given a number N in base r having n digits, the (r-1)’s complement of N is defined as

( rn-1 ) – r.

- Floating point representation uses mantissa and an exponent part  of radix R  .

Answers: ( 1).           false       (2)   true    (3)   true          (4)   false     (5)   false

(6)    true              (7)    false              (8) false        (9)false      (10)   true

Review Questions

- Objective type of questions(Very short notes)<Minimum of ten>

- Analytical type questions<Minimum of ten>

- Essay type Questions<As per requirements>

- Problems<As per required Number>

- Case study<As per required Number>

Skill Building Exercises/Assignments

    - Take the mother board of a computer and identify CPU, memory, peripheral ICs, BUS etc.

    - Buy the components of a computer, assemble, install the software and make it to function.

Eg:-	-Prepare a model of something

-Trace something

-Prepare a report on something etc.,

Previous Questions (Asked by JNTUK from the concerned Unit)

GATE Questions (Where relevant)

Subject is not in gate syllabus

Interview questions (which are frequently asked in a Technical round-Placements)

Real-Word (Live) Examples/Case studies wherever applicable

- List out the intel CPUs in various generation with their specifications. Write how the performance was improved in each generation.

Suggested “Expert Guest Lectures” (both from in and outside of the campus)

Literature references of Relevant NPTEL Videos/Web/You Tube videos etc.

Reference Text Books / with Journals Chapters etc.

T1: M. Moris Mano (2006), Computer System Architecture, 3rd edition, Pearson/PHI, India.

T2: Carl Hamacher, ZvonksVranesic, SafeaZaky (2002), Computer Organization, 5th edition, McGraw Hill, New Delhi, India

R1: Computer Organization Architecture- William Stallings (2006), 7th edition, PHI/PEARSON.

Unit – II

Lectures for the concerned Unit:

Lecture-1

Lecture-2

Lecture-3

Lecture-4

Lecture-5

Lecture-6

Lecture-7

Lecture-8

Lecture-9

Lecture-10

Lecture-11

Test Questions

Fill in the blanks using true or false.

- The operations executed on data stored on memory is called Micro operations.

- The symbolic notation used to describe  the micro operation transfers among registers is called register transfer language.

- The control function is included in the  RTL as:   P    R2            R1.                                   .

- Arithmetic micro operations perform arithmetic operations on binary data stored in registers.

- The AND micro operation can be used to selectively set bits of a register.

- The EX-OR micro operation can be used to selectively complement bits of a register.                                  .

- The AND micro operation can be used for selectively clearing bits in a register.

- The instruction read from memory is placed in the register PC .

- Effective address is defined as the address of the operand  in a computation type instruction or the target address in a branch type instruction.

- Inn hardwired control, control logic is implemented with gates, flip-flops, decoders,and other digital circuits.

Answers:   (1)  True   (2)  True    (3)  false      (4)  false    (5)  false    (6) True      (7)  True

(8) false         (9)  True      (10) True.

- Multiple choice questions<Minimum of ten>

- UNIT-II

- 1. In Reverse Polish notation, expression A*B+C*D is written as

- (A) AB*CD*+ (B) A*BCD*+ (C) AB*CD+* (D) A*B*CD+

- Ans: A

- 2. The addressing mode used in an instruction of the form ADD X Y, is

- (A) Absolute (B) indirect (C) index (D) none of these

- Ans: C

- 3. The BSA instruction is

- (A) Branch and store accumulator (B) Branch and save return address

- (C) Branch and shift address (D) Branch and show accumulator

- Ans: B

- 4. In a program using subroutine call instruction, it is necessary

- (A) initialise program counter (B) Clear the accumulator

- (C) Reset the microprocessor (D) Clear the instruction register

- Ans: D

- 5. A Stack-organised Computer uses instruction of

- (A) Indirect addressing (B) Two-addressing (C) Zero addressing (D) Index addressing

- Ans: C

- 6. Logic X-OR operation of (4ACO) H & (B53F) H results

- (A) AACB (B) 0000 (C) FFFF (D) ABCD

- Ans: C

- 7. When CPU is executing a Program that is part of the Operating System, it is said to be in (A) Interrupt mode (B) System mode (C) Half mode (D) Simplex mode

- Ans: B

- 8. A three input NOR gate gives logic high output only when

- (A) one input is high (B) one input is low

- (C) two input are low (D) all input are high

- Ans: D

- 9. n bits in operation code imply that there are ___________ possible distinct operators (A) 2n (B) 2n (C) n/2 (D) n2

- Ans: B

- 10. The instruction ‘ORG O’ is a

- (A) Machine Instruction. (B) Pseudo instruction.

- (C) High level instruction. (D) Memory instruction.

- Ans: B

c.True or False questions<Minimum of ten>

Fill in the blanks with true or false statement.

- An instruction code is a group of bytes that instruct the computer to perform a specific operation.                                   .

- The number of bits required for the operation code  of an instruction depends on the total number of operations  available in the computer.                                            .

- When the second part of an instruction code specifies an operand, the instruction is said to have direct address                                 .

- If the memory address register has 12 bits, then the program counter register will have 16 bits.                .

- If the load input of a register is enabled, then it will receive data from the bus during the next clock pulse transition.

- The timing signals to the control logic can be derived by decoding the output of a sequence counter.

- The operation of deletion in a stack is called push or push down operation.

- Arithmetic, logical and shift instructions come under data manipulation instructions.

- The instruction that transfers program control  to a subroutine is known as branch and save address.

- Interrupts are classified as traps and faults.

Answer: (1) false    (2) True      (3) false     (4) false       (5)true      (6) true     (7) false    (8)  true

(9)  true                      (10) false

    - Review Questions

- Objective type of questions(Very short notes)<Minimum of ten>

- What do you mean by RTL?.

- What do you mean by common bus system?

- What is the use of 3 state buffers?

- List out different arithmetic operations.

- What do you mean by stored program organization.

- List out registers for a basic computer.

- What is meant by hardwired control unit?

- What is meant by micro programmed control?

- What do you mean by interrupt based data transfer?

- What is reverse polish notation?

b.Analytical type questions<Minimum of ten>

- Referring to the bus system shown above, explain why each of the following micro operations cannot be executed during a single clock pulse. Specify a sequence of micro operations that will perform the operation.

A.         IR            M[PC]            B. AC            AC +TR

C. DR           DR  + AC  ( AC does not change).

- Referring to the above bus system, the following control inputs are active. For each case , specify the register transfer that will be executed during the next clock transition.

|  | S2 | S1 | S0 | LD of register | Memory | Adder |
| --- | --- | --- | --- | --- | --- | --- |
| a. | 1 | 1 | 1 | IR | Read |  |
| b. | 1 | 1 | 0 | PC |  |  |
| c. | 1 | 0 | 0 | DR | Write |  |
| d. | 0 | 0 | 0 | AC |  | Add |

- The following register transfers are to be executed in the above system. For each transfer specify: (1) The binary value that must be applied to bus select inputs S2, S1, S0; (2)the register whose load control input must be active(if any); (3) A memory read or write operation ( if needed); and (4) the operation in the adder and logic circuit (if any).

8.

9.

10.

- Essay type Questions<As per requirements>

Essay Type Questions:

UNIT-II

- What is the need of addressing modes? Explain different types of addressing modes

- List out the instruction formats used in the processor and discuss with example

- Explain about machine instruction characteristics

- What are the differences between direct and indirect addressing instructions? How

many references to memory are needed for each type of instruction to bring an operandin to a process register?

- Discuss about the register organization in computer

- Explain about registers for floating point arithmetic operation

- Discuss about one stage of a decimal arithmetic unit.

- Discuss about adding of decimal numbers methods.

- Discuss about the design of the control unit

- ExplainBooth’salgorithm. ApplyBooth’salgorithm tomultiplythetwodecimal numbers14and12.Assumethemultiplierandmultiplicandtobeof5bitseach

- Problems<As per required Number> N.A

- Case study<As per required Number>N.A

Skill Building Exercises/Assignments

    - Compare Instruction set of 8 bit and 16 bit  and 32 bit processors

    - Compare addressing mode of 8 bit, 16 bit and 32 bit processors.

    - Compare features of 8,16 and 32 bit processors

Eg:- 	-Prepare a model of something

-Trace something

-Prepare a report on something etc.,

Previous Questions (Asked by JNTUK from the concerned Unit)

- A. Consider 4 , 4-bit registers A,B,C and D connected to a common bus system using multiplexers. What has to be done to the bus system so that information can be transferred from any register to any other register?.

B. Represent the following conditional control statements by two register transfer statements with control functions. If ( p=1 ) then ( R1          R2) else if ( Q =1 ) then ( R1        R3 ).

2. A digital computer has common bus system for 16 registers of 32 bits each. The bus is constructed with multiplexers.

a. How many selection inputs are there in each multiplexer?

b. What size of multiplexers are needed?

c. How many multiplexers are in the bus?

X +yz:      AR                    AR + BR

3a.Draw the block diagram of the hardware that implements the following statements:

,  where AR and BR are 2 n bit registers and x, y and z are control variables. Include the logic gates for the control function.

b. Show the hardware that implements the following statements.Include the logic gates for the control function and a block diagram for the binary counter with a count enable input.

xyT0  + T1 +y’ T2:     AR                    AR +1   .

4. Design a 4 bit combinational circuit decrementer  using 4 full adder circuits.

5. What is wrong with the following register transfer statements?

- xT:  AR   AR ,   AR   0

- yT:   R1   R2,  R1    R3

- zT:   PC    AR,  PC    PC + 1

6. (a) Explain the various Instruction types.

(b) Draw and explain the flow chart for instruction cycle. 7 (a) Explain various instruction formats with examples. (b) Write short notes on process organization. 8. (a) List and explain the characteristics of machine instructions (b) Describe various addressing modes in detail (c) Give a short note on instruction pipelining 9a. Draw the flowchart for memory reference instructions and explain. b. Discuss about stack organization with register stack and memory stack. 10a. Design a combinational circuit for i) 4-bit shifter ii) 4-bit decrementer and explain.

b.  Briefly explain the computer registers for basic computer.

11a. Design a combinational circuit for the following arithmetic operations

- Addition (ii) Subtraction (iii) Increment (iv) decrement

b. Discuss about instruction codes and stored program organization

Interview questions (which are frequently asked in a Technical round-Placements)

4.2.10       Interview questions

1. What is micro operation in a digital computer.

2. What is register transfer language?

3. What do you mean by addressing mode?

4. What is indirect address?

5. What is meant by instruction set of a computer?

6. What is the use of PC and IR in a digital computer?

7. What is stack in a computer?

8. Explain instruction format.

9. What is interrupts and interrupt service routine?

10. What is RISC and CISC?

Unit – III

Lecture-1

Lecture-2

Lecture-3

Lecture-4

Test Questions

- Fill in the blanks type of questions

Fill in the blanks type questions.

- The function of control unit in a digital computer is to initiate

Of micro operations.

- When the control signals are generated by hardware using conventional                                 techniques , the control unit is said to be hardwired.

- The control  variables at any given time can be represented by a string of

Called a control word.

- A control unit whose                                      control variables are stored in memory is called a micro programmed control unit.

- A                                                             of micro instructions  constitutes a micro program.

- The next address generator in a control unit is  sometimes called micro program

- The data register in a micro programmed control unit is sometimes called

- .The transformation from the instruction code bits to an address in control memory where the routine is located is referred to as a                                            .

- are programs that are used by other routines to accomplish a particular task.

- In a microinstruction format , the CD field select                                    bit conditions.

- Multiple choice questions

1. Translation from symbolic program into Binary is done in

(A) Two passes. (B) Directly (C) Three passes. (D) Four passes.

Ans: A

2.A microprogram sequencer

(A) generates the address of next micro instruction to be executed.

(B) generates the control signals to execute a microinstruction.

(C) sequentially averages all microinstructions in the control memory.

(D) enables the efficient handling of a micro program subroutine.

Ans: A

3.The operation executed on data stored in registers is called

(A) Macro-operation (B) Micro-operation

(C) Bit-operation (D) Byte-operation

Ans: B

4. Microinstructions are stored in control memory groups, with each group specifying a

(A) Routine (B) Subroutine (C) Vector (D) Address

Ans: A

5. PSW is saved in stack when there is a

(A) interrupt recognised (B) execution of RST instruction

(C) Execution of CALL instruction (D) All of these

Ans: A

6. In a vectored interrupt.

(A) the branch address is assigned to a fixed location in memory.

(B) the interrupting source supplies the branch information to the processor through an interrupt vector.

(C) the branch address is obtained from a register in the processor

(D) none of the above

Ans: B

7. The communication between the components in a microcomputer takes place via the address

and

(A) I/O bus (B) Data bus (C) Address bus (D) Control lines

Ans: B

8. In a program using subroutine call instruction, it is necessary

(A) initialise program counter (B) Clear the accumulator

(C) Reset the microprocessor (D) Clear the instruction register

Ans: D

9. A microprogram written as string of 0's and 1's is a a. symbolic microinstruction b. binary microinstruction c. symbolic microprogram d. binary microprogram

Ans d

c. fill the blanks with true of false statement.

- A word in control memory location is called microinstruction.                                                 .

- A group of microinstructions constitute micro program.                                             .

- In dynamic microprogramming, the micro program  can be initially loaded from disk.

- Each machine instruction initiate a microinstruction in control memory.

- The microinstruction specifies various internal control signals  for execution  of register micro operations.                                                 .

- Each machine instruction has associated micro program in control memory.

- While the micro operations are are being executed, the next address is computed in the next address generator circuit and then transferred into the control data  register.

- If we want to establish a different control  sequence for the system,  we need to simply replace the control ROM with suitable micro programs , in the control unit of micro programmed control unit.

- The subroutine register in the control unit stores the starting address of the subroutine.

- Some of the bits in the instruction code is used to reach the starting address of the micro program routine associated with that instruction and this process is called mapping of instruction.                                     .

Answer:

(1) True       (2) false  because a sequence of microinstructions constitute  a microprogram.

( 3) True    (4) false because it initiates a series of microinstructions.

(5)  True     (6) True       (7) false    (8)True     (9) false    9100 True

Review Questions

- Objective type of questions(Very short notes)

- What is the difference between a microprocessor and a micro program?

- Is it possible to design a micro program without a micro program?

- Are all micro programmed computers also microprocessors?

- Is it possible to have hardwired control associated with a control memory?

- Explain the difference between hardwired control and micro programmed control.

- Te4ll the mapping process from instruction code to microinstruction address using ROM.

- Specify a minimum of 3 components used in a micro program sequencer.

- What is the advantage of writable control memory?

- What is the function of CD and AD fields in the Microinstruction Format ?

- Hat is the use of program counter.

- Analytical type questions<Minimum of ten>

- With reference to the above figure, the propagation delay timesare : 1)  to generate the next address is 40ns, 2) to transfer the address into the control address register 10ns, 3) to access the control memory ROM 40 ns, 4) to transfer the microinstruction into the control data register 10ns, 5) to perform the required micro operation specified by the control word 40ns. What is the maximum clock frequency that the control can use?.

- Formulate a mapping procedure that provide 8 consecutive microinstructions for each routine. The operation code has 6 bits and the control memory has 2048 words.

- Show how a 9-bit micro operation field in a microinstruction can be divided into subfields to specify 46 micro operations. How many micro operations can be specified in one micro instruction?.

- A computer has 16 registers, an ALU with 32 operations, and a shifter with 8 operations, all connected to a common bus system.

- Formulate a control word for a micro operation.

- Specify the number of bits in each field of the control word and give a general encoding scheme.

- Show the bits of the control word that specify the micro operation  R4           R5 + R6.

- The control memory system shown above uses a control memory of size 1024*32. The microinstruction has 3 fields as shown in the diagram. The micro operation field has 16 bits.

- How many bits are there in the branch address field and the select field?

- If there are 16 status bits in the system, how many bits of the branch logic are used to select a status bit?

- How many bits are left to select an input for the multiplexer?

- The control memory shown above has 4096 words of 24 bits each.

- How many bits are there in the control address  register?

- How many bits are there in each of the 4 inputs shown going into the multiplexer?

- What are the number of inputs in each multiplexer and how many multiplexers are needed?

- Which control unit you will prefer? Tell the reasons for your decision.

The micro operation for BSA  instruction is given above and the microinstruction format is also given above. Write the symbolic micro program routines for the BSA instruction.

- Essay Type Questions:

- Explain about the micro programmed control organization

- Explain the differences between hardwired control and micro programmed

Control.

- Define following terms, micro operations, microinstruction, micro program,

micro code.

- Explainthevarietyoftechniquesavailableforsequencingofmicroinstructions basedontheformatoftheaddressinformationinthemicroinstruction.

- ExplaintheOrganization ofthecontrolunittoallowconditionalbranchinginthe micro program

- Explain how the mapping from an instruction code to a micro instruction address can be done by means of ROM.

- Draw the block diagram of a micro programmed sequencer and explain its operation.

- Explain thebasic organizationof amicroprogrammedcontrol unitandthegenerationofcontrolsignalsusingmicroprogram.

- Problems

- Using the mapping procedure given above, give the first microinstruction address  for the following operation code: (a) 0010; (b) 1011; (c) 1111.

- Explain how the mapping from an instruction code to microinstruction address can be done by means of a ROM. What is the advantage of this method compared to the method shown in the figure above?.

- Case study<As per required Number> N.A.

Skill Building Exercises/Assignments

Draw a neat chart of micro program sequencer.

Write a report on the functioning of plcs

Previous Questions (Asked by JNTUK from the concerned Unit)

JNTUK QUESTIONSJ

1.   Discussthebasicorganizationofamicroprogrammedcontrolunitandthegenerationof controlsignalsusingmicroprogram

2.  Explaintheconceptofmicroprogrammedcontrolunit.

3.  (a)Explainthevarietyoftechniquesavailableforsequencingofmicroinstructions basedontheformatoftheaddressinformationinthemicroinstruction.

(b)CompareandContrasthardwiredcontrolunitwithmicroprogrammedcontrolunit.

4.  (a)Hardwiredcontrolunitisfasterthanmicro programmedcontrolunit?Justify thisstatement.

(b)Briefly explainthebasic organizationofamicro programmedcontrolunitandthe generationofcontrolsignalsusingmicroprogram

5 a)  Explaintheexecutionofmicroinstructionswithaneatdiagram.   [5]

b)  Brieflydescribethedesignofahardwiredcontrolunit. [10]

6. Explainthedesignofmicro-programmedcontrolunitindetail. [15]

7 a)  ExplaintheOrganization ofthecontrolunittoallowconditionalbranchinginthe microprogram.

b)  Whatishardwiredcontrol?Howisitdifferentfrommicroprogrammedcontrol?

8.Explain thebasic organizationof a micro programmed  control unit and the generationofcontrolsignalsusingmicroprogram.

9. a)Givethetypicalhorizontalandverticalmicroinstructionformats.

b)Describehowmicroinstructionsarearrangedincontrolmemoryandhowthey areinterpreted.

10.a)Clearlydistinguishbetween

i.   Packed/Unpackedmicroinstructions   ii. Hard/Softmicroprogramming

b)Listandbrieflyexplainapplicationsofmicroprogramming.

11.a)Whatarethedesigngoalsforadesignerwhiledecidingahardwiredormicro programmedCUforaCPU.

b)Writeshortnoteson microinstructionsequencing.

12.	a) Definethefollowing:

i.   Microoperation

ii. Microinstruction

iii.Microprogram

iv. Controlmemory.

b) Explaintheselectionofaddressforcontrolmemory?

13.a)GivetheCircuitDiagramforMicroProgramSequencefora ControlMemory.

b)Whatisthedifferencebetweena MicroprocessorandaMicroProgram?           [8M+8M]

14.a)DiscussindetailaboutAddressSequencing.

b)WritetheSymbolicMicroProgramforBranchandSaveInstructions. [8M+8M]

15.a) DiscussindetailaboutthedecodingofMicroOperationfields.

b) Showhowa9-bitMicrooperationfieldinMicroinstructioncanbedividedintosubfields to specify46 Microoperations?HowmanyMicrooperationscanbespecifiedinone Microinstruction? [8M+8M]

16.a) WhatistheDifferencebetweenaMicroprocessorandaMicroprogram?Isitpossibleto designaMicroprocessorwithoutaMicroprogram?

b)WhatisthefunctionofControlUnit?ExplainthedifferencebetweenHardwiredcontrol andMicroprogrammedcontrol. [8M+8M]

17 a) Explain briefly about the micro programmed control organization. [7]

b) Briefly discuss about the design of the control unit[8]

18. a) Discuss about the process organization in computer.

b) Discuss about the register organization in computer.

19. a) Define following terms, micro operations, microinstruction, micro program,      	micro code.

b) Explain the differences between hardwired control and micro programmed

control.

20. a) Draw the block diagram of microprogram control organization and explain.

b) Discuss how selection of address for control memory.

GATE Questions (Where relevant)

N.A.

Interview questions (which are frequently asked in a Technical round-Placements)

  - What is micro operation and Micro program?.

  - What is the use of control memory in a microprogrammed control?

  - What is meant by hardwired control?

  - What are the advantages of micro programmed control unit?

  - What are the functions of a micro program sequencer?

Real-Word (Live) Examples/Case studies wherever applicable

- Micro programmed State Machine design.

- A lot of machine controls with sequential operations can be implemented with control memory. Ex: Automatic machine tools with sequential control

Suggested “Expert Guest Lectures” (both from in and outside of the campus)

Literature references of Relevant NPTEL Videos/Web/You Tube videos etc.

http://nptel.ac.in/video.php?subjectId=106102062

For the following Topics: 	Processor Design Micro programmed Control (33:11)

Any Lab requirements; if so link it to Lab Lesson Plan.

N.A.

Reference Text Books / with Journals Chapters etc.

Text Book:

M. Moris Mano (2006), Computer System Architecture, 3rd edition, Pearson/PHI, India.